# Booster Robocup_Demo — Documentation Summary

Group 75 — RoboCup Humanoid Soccer League — Sprint 2, Week 1 (T5)

Prepared by: Sayed Isfaque Ahmed Anan (BA)

This summarises the official Booster robocup_demo documentation (github.com/BoosterRobotics/robocup_demo), scoped to what's needed for our Week 1 tasks: getting the demo running in simulation, and calling it directly to confirm the robot walks.

# 1. What the demo actually does

The Booster K1/T1 official RoboCup demo lets the robot make autonomous decisions — detect the ball, decide what to do, and physically act — to complete a full match on its own. It's built from three separate programs:

| Program | What it does |
| --- | --- |
| vision | Detects the ball, other robots, and the field using a Yolo-v8 model; calculates their position relative to the robot |
| brain | The decision-maker — reads vision data plus game-control data, decides what the robot should do, and controls it to act |
| game_controller | Listens for referee data packets on the local network and converts them into a format brain can use |

# 2. Two ways to run it

- Virtual robot (simulation) — via ./scripts/sim_start.sh. This is our path for Sprint 2, since we don't have physical hardware.

- Real robot — via ./scripts/start.sh. Not relevant to us right now.

# 3. Setup

## Dependencies

- ros-humble-backward-ros (install via apt-get)

- onnxruntime — required if building without CUDA (our likely case, since we're not on an NVIDIA GPU setup)

## Build

Two build scripts exist — use the CPU-only one unless we have CUDA available:

./scripts/build_no_cuda.sh     # CPU-only (our case)

./scripts/build.sh             # Needs CUDA — real robot only

## Known gotcha — flag this before it costs time

If the repo is cloned on Windows and the build is run inside the simulator, Windows line-endings break the build script with a "bad interpreter" error. Documented fix, run before building:

cd booster_soccer/scripts
find . -type f -print0 | xargs -0 sed -i 's/\r$//'

# 4. Config —

For simulation mode specifically, two config files need updating before running:

| File | What to set |
| --- | --- |
| src/brain/config/config.yaml (or config_local.yaml) | Point vision.image_camera_info_topic, vision.depth_image_topic, vision.depth_camera_info_topic at the simulator's own topics |
| src/vision/config/vision.yaml (or vision_local.yaml) | Set camera.color_topic / depth_topic / intrin_topic to match the sim; set detection_model.model_path to ./src/vision/model/sim_data_det_0126.onnx and segmentation_model.model_path to ./src/vision/model/sim_data_seg_0126.onnx — these are the simulation-specific model files, not the real-robot ones |

# 5. Running it once built

./scripts/sim_start.sh         # simulation — our target for T2/T3/T4

Once this runs cleanly, that satisfies T2 ("confirm vision, brain, game_controller all live"). From there, Mohammed's T3 task is to call the Booster API directly with a standalone script and confirm the robot walks — the vision/brain/game_controller stack above is what that script will be talking to.

# 6. Things to be aware of, not needed yet

- RLVisionKick.enableAutoVisualKick — an auto-kick feature. K1 hardware only, needs a specific firmware version. Doesn't apply to our simulation work — don't spend time trying to enable it.

- There's an active branch (sandbox/support_2026_game_controller) with 2026 rule updates, but the repo's own notice says it isn't fully tested and penalty-kick logic isn't implemented. We should stay on the main branch unless we have a specific reason to switch.

- The current open-source code doesn't yet comply with the newest rule restricting robot-to-robot communication. Not relevant to our integration work, but worth knowing if it comes up with Timothy.